This is how a computer is able to process complex tasks:

The CPU is using logical gates to process complex tasks. The CPU consists of billions of logical gates that are interconnected which each other.

By combining gate types like AND, NAND, OR, NOR, NOT, XOR and XNOR engineers build small circuits (like adders or memory cells). These circuits are then combined into larger units like the ALU (Arithmetic Logic Unit) which together let the CPU execute complex instructions.

We as programmers don´t need to understand the different circuits.

If you want to know how a adder works then go to this website:

https://www.geeksforgeeks.org/digital-logic/binary-adder-witQh-logic-gates/

What we as programmers need to understand

Logical binary operators:

AND, NAND, OR, NOR, NOT, XOR, XNOR

In the next section I will show some practical examples. Lets talk at first about Truth tabels:

| Logical AND |  |  |
| --- | --- | --- |
| A | B | Result |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

Negate the results of a logical AND and then you have the results for a NAND.

| Logical NAND |  |  |
| --- | --- | --- |
| A | B | Result |
| 0 | 0 | 1 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

| Logical OR |  |  |
| --- | --- | --- |
| A | B | Result |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

| Logical NOR |  |  |
| --- | --- | --- |
| A | B | Result |
| 0 | 0 | 1 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 0 |

The logical NOT negates the value in column A:

| Logical NOT |  |
| --- | --- |
| A | Result |
| 0 | 1 |
| 1 | 0 |

Negate the results of a logical XOR and then you have the results for an XNOR.

| Logical XOR |  |  |
| --- | --- | --- |
| A | B | Result |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

| Logical XNOR |  |  |
| --- | --- | --- |
| A | B | Result |
| 0 | 0 | 1 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

How to use logical binary operators to check if a number is positive or negative

1 byte = 8 bit

Most signifcant bit
Least significant bit

Signed byte:
highest possible value: 127

In binary: 0111 1111

lowest possible value: -128

In binary: 1000 0000

0100 0110 (70)

1000 0000 (-128)

0000 0000 Result

70 AND -128 = 0

1011 1010 (-70)

1000 0000 (-128)

1000 0000 Result
-70 AND -128 = -128

XOR encryption and decryption

65 = 0100 0001

Encryption key = 123

0100 0001 (65)

0111 1011 (123 encryption key)

0011 1010 Result (58)

0011 1010 (58)

0111 1011 (123 encryption key)

0100 0001 Result (65)

In a later video I will explain how to use XOR to encrypt and decrypt strings in a program.