The General-Purpose Register (GPR-Register)

A CPU register is build of digital logic circuits. A CPU register can be used to store data. Its extremly fast and reading or writing data to a register is way faster than to a memory address.

If we want to read or write data from RAM for example then the data needs to go over a bus from the RAM to the CPU and that costs time. A bus is something like a physical wire that is used to transfer data from A to B or from B to A.

But the size of data that we can save in CPU register is pretty limited.

Register	Purpose			Explanation

RAX 		Accumulator 			Used for arithmetic results and function return values.

RBX 		Base				Functions will not change the value of this register.

RCX 		Counter 			Used for counting operations like REP or LOOP.

RDX 		Data 				Its holding the high order bits in multiplications and

divisons. Its also used in register pairing (RDX:RAX)

RSI 		Source index 			Used as a source register for string operations.

RDI 		Destination index 		Used as a destination register for string operations.

RBP 		Base pointer (stack frame)	Base of the current stack frame. That is used to

reference local variables.

RSP 		Stack pointer 			This points to the top of the stack.

R8 		General purpose

R9	 	General purpose

R10 		General purpose

R11 		General purpose

R12 		General purpose

R13 		General purpose

R14 		General purpose

R15 		General purpose

These registers will become very important when we talk about API calls:

RDI, RSI, RDX, RCX, R8, R9

64 Bit 		32 Bit 		16 Bit 		8 bit high-part 		8 Bit low-part

RAX			EAX			AX			AH				AL

RBX			EBX			BX			BH				BL

RCX 			ECX 			CX 			CH 				CL

RDX 			EDX 			DX 			DH 				DL

RSI 			ESI 			SI 			- 				SIL

RDI 			EDI 			DI			- 				DIL

RBP 			EBP 			BP 			- 				BPL

RSP 			ESP 			SP 			-				SPL
R8 			R8D 			R8W 			- 				R8B

R9 			R9D 			R9W	    		- 				R9B

R10 			R10D 			R10W 			- 				R10B

R11 			R11D 			R11W 			- 				R11B

R12 			R12D 			R12W 			- 				R12B

R13 			R13D 			R13W 			- 				R13B
R14 			R14D 			R14W 			- 				R14B

R15 			R15D 			R15W 			- 				R15B

EAX, AX, AL and AH are inside of RAX.

Example:

64 Bit

RAX: 0000000000000000

EAX

RAX: 0000000000000000

The blue area is the high part of RAX and the green part is the low part of RAX.

Exercise

Where is AX?

RAX: 0000000000000000

Where is AH?

RAX: 0000000000000000

Where is AL?

RAX: 0000000000000000

Hint:

RAX: 0000000000000000

Each colored number represents 1 byte. (1 byte = 8 bit). It doesn’t matter if a colored value is 00 or ff both values have a size of 1 byte.

At the end of the video I will show the solution.

The RFLAG Register

The RFLAG register is a 64 bit register that records the outcome of a logical or arithmetic operation.

These are the most important bit flags for us:

ZF = Zero Flag

OF = Overflow Flag

CF = Carry Flag

SF = Sign Flag

DI = Direction Flag

Later in the course I will explain how to use the 5 most important flags.

Example for a logical operation:

rbx = 1
rcx = 1

cmp rbx, rcx

cmp means compare. It compares if rbx holds the same value as rcx.

If both register hold the same value then the ZF (zero flag) register gets the value 1 otherwise ZF gets the value 0.

Example for a arithmetic operation:

rbx = 1

rcx = 6
sub rbx, rcx

sub means subtract. In this case it subtracts the value in rbx from the value in rcx.

1 – 6 = -5

If the result of the substraction is negative then the SF (sign flag) gets the value 1. If the result of the substraction is positive then the SF flag gets the value 0.

A flag overview from AMD:

More informations about the flags can be found here:

AMD64 Architecture Programmer's Manual Volume 1: Application Programming:
Page: 70

https://docs.amd.com/v/u/en-US/24592_3.24

AMD64 Architecture Programmer's Manual Volume 2: System Programming
Page: 52

https://docs.amd.com/v/u/en-US/24593_3.44_APM_Vol2

More register:

Solution to the excercises:

Where is AX?

AX

RAX: 0000000000000000

Where is AH and where is AL?

AHAL

RAX: 0000000000000000